All of these example file use the Atmel Devices ATF1500, ATF1508, ATV5000, ATV2500b and ATV750b

## \CE to T FlipFlop\CE_TFF2.PLD

DEVICES:    F1508PLCC84

This example shows how to use the ATF15XX family registers clock enable to simulate a T-Flip Flop.

## \Clock\CLOCK.PLD

DEVICES:    F1500

This example shows 3 different clocking methods for the ATF15XX family.

## \16 bit Counter\CNT16.PLD

DEVICES:    F1500

16-bit Loadable Counter with Clock Enable and Reset.

## \Property 1\COMREG.PLD

DEVICES:    F1508PLCC84

This example demonstrates the Atmel Properties JTAG, and LOW_POWER=ON.

## \Frame Grabber Controller\CONTROL.PLD

DEVICES:    V2500

Frame Grabber Controller for the ATV2500 device

## \Clock and DT type FF conversion\CONVERT.PLD

DEVICES:    F1500

Example showing clock usage and D/T type FF conversion

Using the REGISTER_SELECT keyword

## \20 bit Counter\EX375.PLD

DEVICES:    v750

20 Bit Counter for ATMEL V750

Simulation not Supported.

## \Expander Cell\EXPANDER.PLD

DEVICES:    F1500

Example assigning expander terms.

Simulation not Supported.

## \Fast Input Latch\FASTLAT.PLD

DEVICES:    F1508PLCC84

This example demonstrates the fast input latch feature on the ATF1508

## \8 bit Pseudo Grey Code Counter and Decode\GCOUNT.PLD

DEVICES:    V750

8-Bit Pseudo Grey Code Counter and decode

This example is of a Mealy state machine - in other words,

combinatorial outputs are generated from the machine states and the

combinatorial inputs.  This is actually a very common design -

decoding the state machine's outputs into combinatorial outputs.

Problems can arise when pure binary counters are used, as many bits

are changing simultaneously.  Race conditions can result and

glitches appear on combinatorial outputs when many inputs change

simultaneously.  We recommend using a Grey coded counter.  However,

it is easier to define a large counter with Boolean equations than

with a state machine diagram.  This example is a pseudo Grey code

counter for 8 bits.  Smaller counters can be cut down from this one

and larger ones can be built up using the same algorithm.

## \Product Term Joining\JOIN2500.PLD

DEVICES:    V2500lcc

Example of term joining using the ATMEL 2500.

## \Traffic Signal Controller\MTRAFSIG.PLD

DEVICES:    V750BLCC

Traffic Signal Controller.

## \2 to 1 Mux\MUX.PLD

DEVICES:    F1500

Simple combinatorial logic: 2-to-1 MUX

## \Pinkeep Controller\PINKEEP.PLD

DEVICES:    F1500AT

Select ATF1500 Fitter Property list

1) Preassign Keep is on - Keep User assigned pins

2) Soft_buffer is on - Node Collapsing is Disabled

Note: all options separated by the <space> character

3) Disable Pin Keeper Circuits

4) Enable Security Bit

Note: Options 3 and 4 are only available on the ATF1500A Device. This

device allow programmable control of the pin-keeper circuits and the

security bit feature. Selecting the f1500at defaults to having the

security bit disabled and the pin-keeper circuits enabled

Simulation Not Supported

## \Property 2\PROPERTY.PLD

The purpose of this example is to show the user how to use the property

statement.

Simulation not supported.

## \Reset\RESET.PLD

DEVICES:    F1500

Example showing reset usage

Simulation not Supported.

## \Register and Latch\REGLATCH.PLD

DEVICES:    F1500

Example showing register and latch usage

## \16 bit Loadable Shift Register\SHIFT16.PLD

DEVICES:    F1500

16-Bit Loadable Shift Register.

## \8 bit Magnitude Comparator\T885.PLD

DEVICES:    V750BLCC

The 74885 is a 8-bit magnitude comparator which can be cascaded without other circuitry.  The 'P' inputs are latched into 8 flip-flops to become the 'Q' value.  Until a new value is latched into the 'Q' registers, all values  on the 'P' inputs are compared to the current 'Q' values. This design also includes an 'Equal' output and input.

## \16 bit Timer Counter\TIM_CNT.PLD

DEVICES:    F1508PLCC84

The 16-bit design is broken up into 4 parts, each a 4-bit design. Except for the COUNTER module, all modules can easily be seprated into 4 parts. As for the COUNTER design, CARRY-OUT functions are created to cascade the four 4-bit counters. We will be using synchronous carry-out functions which looks ahead so to improve the performance of the design.