All of these examples relate to popular simple PLD’s that are currently available on the Market

# G16V8

## \4 Bit Asynch Adder\ADDER.PLD

DEVICES:    GAL16V8, PAL16L8, PAL16P8, 82S153

4-bit asynchronous adder implemented as a ripple-carry through four adder-slice circuits.  Each adder-slice was implemented using a user-defined Function.

## \4 Bit Synch Counter With Clear\COUNT10.PLD

DEVICES:    GAL16V8, PAL16RP4, GAL16V8

4-bit up/down decade counter with synchronous clear capability.  An asynchronous ripple carry output is provided for cascading multiple devices.

## \DType FlipFlip\FLOPS.PLD

DEVICES:    GAL16V8, PAL16P8, PAL16RP8, 82S159

Using D-type flip-flop to create a 2-bit counter (four ways).

## \Simple Gates\GATES.PLD

DEVICES:    GAL16V8, PAL16L8, PAL16P8, 82S153

Simple use of NOT, AND, OR, and XOR gates.

## \IO Decode\IODECODE.PLD

DEVICES:    GAL16V8, PAL12L6, PAL12P6, 82S153

A chip select signal generator for I/O functions.  It also enables the

data bus transceiver for both memory and I/O write cycles.

## \Memory Decode\MDECODE.PLD

DEVICES:    GAL16V8, PAL16L8, PAL16P8, 82S153

A memory RAS signal generator and CAS signal initiator.  It also enables

the data bus transceiver for both memory and I/O read cycles.

## \Chip Select\WGTTL.PLD

DEVICES:    GAL16V8, 82S155

This design is similar to the previous example with the exception that it implements the logic as TTL macros.

# GAL22V10

## \Lookup Table\LOOKUP.PLD

DEVICES:   GAL22V10C,  PAL22V10

This is an example of the TABLE format for CUPL.  The lookup  table input consists of a 6 bit number for the radius of a circle (R0-5), the output is the value for the perimeter (2 x Pi x Radius) of the circle (P0-7).

# EP600

## \16 bit UpDown Counter\TCOUNTER.PLD

DEVICES:    EP600

16-bit up/down counter with built-in shift register using toggle flip-flops.

# FPLA

## \ 4 bit UpDown Shift Counter\SHFTCNT4.PLD

DEVICES:    82S159

4-bit counter/shifter using JK-type flip-flops.

# OTHER

## \ 13 bit Counter\COUNT13.PLD

DEVICES:    PAL32R16

13-bit counter using Set notation with parallel load, hold, and clear.

## \ 8 bit Counter v1\COUNT8.PLD

DEVICES:    PAL20X8

8-bit counter with parallel load, clear, and hold using XOR capability.

## \ 8 bit Counter v2\COUNT8A.PLD

DEVICES:    PAL20X8

8-bit counter with parallel load, clear, and hold using Set notation.

## \ IO Port\IOPORT.PLD

DEVICES:    PAL20RA10

7-bit register with handshake logic used to interface between a microprocessor and I/O port.

## \ 8 bit Ripple Counter\RIPPLE8.PLD

DEVICES:    PAL20RA10

8-bit ripple counter with asynchronous load.

# ROM

## \ Adder Truth Table\ADDER_TT.PLD

DEVICES:    RA9P8 (512x8 PROM)

4-bit asynchronous adder implemented using a truth table.  Makes use of nested $Repeat statements.

## \ Hexadecimal Display\HEXDISP.PLD

DEVICES:    RA5P8 (32x8 PROM)

Hexadecimal to 7-segment decoder used for displaying numbers.

## \ Lookup Table\LOOKUP.PLD

DEVICES:    RA8P8 (256x8 EPROM)

Arithmetic lookup table that calculates the perimeter of a circle

given the radius.  Truth Table syntax is used.