Program 5X86.EXE

Author Peter N Moss

E-Mail Address pmoss@yoda.alt.za

Version B9,  20 January 1996.

Source code: Source code is not included.

Compiled with Turbo Pascal V5.5 and TASM V1.1

Files:

5X86xx.ZIP      The combined zipped distribution package

5X86.EXE        The program file

CX5X86.EXE      Device driver

5X86.REG        Registration form

5X86.DOC        This file

The program: 5X86.EXE

The program detects the presence of a 5x86 processor and will not

run if a Cyrix/IBM 5x86 is not found.

5x86.EXE will report and enable the modification of the performance

control register of the Cyrix 5x86 (TM) processor.  Access to register

bits marked reserved (*) is not supported other than to report status

if in some cases non zero.

Usage: 5x86 <[/H|?] [/Q]> </BTB_EN=[on|off] /LOOP_EN=[on|off]

/LSSER=[on|off] /RSTK_EN=[on|off] /BWRT=[on|off]

/WT1=[on|off]  /LINBRST=[on|off] /MEM_BYP=[on|off]

/DTE_EN=[on|off] /FP_FAST=[on|off]> <[/ICD|/CWT|/CWB]> <[/T]>

Where BTB_EN  Branch prediction

LOOP_EN Loop (Prefetch buffer)

LSSER   Load store serialise, enable reorder

RSTK_EN Return stack

BWRT    Burst write cycle 4 dwords on cache line replacement.

WT1     Write through region one (640kb..1Mb).

LINBRST Linear burst mode.

MEM_BYP Memory read bypassing

DTE_EN  Directory table cache

FP_FAST Fast NPU exception reporting

ICD     CPU Internal cache disable

CWT     CPU cache enable, write through mode

CWB     CPU cache enable, write back mode

Q = Program version

H,? = Help

T = Status of PCR0 register

E = (Reserved) Debug report.  Status of PCR0, CCR1..4

and CR0

Note that T may be combined with register bit settings

and will report status after the register has been set

by reading the register. Thus giving confirmation that

the register has been set correctly ie 5x86 /LSSER=on

/BTB_EN=on /RTS=off /t

/E is included for debugging and expansion.  The report

is more than 25 lines so it would be advisable to use

this with either a DOS pipe to more (| more) or

redirection to a file (> filename).  It is is not part

of the program and is not expected to continue beyond

its current use.  Note CR0 NW bit cache status will

always report write through mode regardless of the

actual operating mode.

On = enable, 1; Off = disable, 0.  On and off are valid

and all other values are treated as a error.

The first three characters only need be entered. LIN=on

is the same as LINBRST=on.  Less than three characters

of the register bit name will cause an error.

The cache enable/disable parameters are only valid for

the CPU at level 0.  DOS operates at level 0 (ring 0)

if EMM386 is not loaded.  No checking or testing is

done for ring 0 operation.  The report of CR0 will be

incorrect for cache status as the NW bit does not

reflect cache status.  This is a currently known fault

and will be corrected in a future version.  It is only

reflected using the /e switch which is for debugging

purposes only.

Upper case or lower case characters may be used.  The

order of register input is not important.  H, ? or Q

will terminate any other command line parameters.

T may be placed anywhere on the command line.

Command line input may use any legal DOS separator,

space tab / ; etc.

Undocumented commands added to version B9

/CPUID=[on|off]     on = enable the CPUID command.

/SMAC=[on|off]

/MMAC=[on|off]

/USE_SMI=[on|off]   on = enable SMM pins

/IORT=[on|off]      on = 16 clks, off = 0

Errorlevel:

5 = No Cyrix 5x86.

1 = Command line entry error

0 = Normal termination.

The Cyrix 5x86 CPU registers:

Full details of the register address and access requirements can be

found in the Cyrix 5x86 (TM) processor data book #94192-00.

-oo-

Register: Performance control register 0

The following bits of this register perform various functions detailed

with the register bit name.

Register PRC0 Index 20h

Access to this register is via I/O ports 22h (Index) and 23h (Data)

and the MAP_EN bits of the CCR3 register.

Bit map:

----------------------------------------------------------------

7      6      5      4      3        2        1        0

----------------------------------------------------------------

|LSSER|  Res4|  Res3|  Res2|  Res1|  LOOP_EN|  BTB_EN|  RSTK_EN|

----------------------------------------------------------------

Bit    Name     Function

7      LSSER    1 = Load store serialise enable (Reorder disable)

6      Res4     BTB Test register

5      Res3     Reserved

4      Res2     Reordering of locked misalighned loads

3      Res1     All instructions stalled to serialise pipeline

2      LOOP_EN  Loop enable

1      BTB_EN   Branch target buffer enable

0      RSTK_EN  Return stack enable

RSTK_EN: 1 = The return stack is enabled and RET instructions will

speculatively execute the code following the associated CALL

to improve performance.

BTB_EN:  1 = Branch target buffer enable.  Branch prediction occurs.

LOOP_EN: 1 = The CPU will not flush the prefetch buffer if the

destination jump is already in the prefetch buffer

eliminating the need for a read from the cache.

LSSER:   1 = All memory reads and writes will occur in execution

order. Reordering disabled.

LSSER should be set to ensure that memory mapped I/O devices

operating outside of the address range 640k..1M will operate

correctly.

Memory accesses in the range 640k..1M will always be issued

in execution order.

-oo-

Register: Configuration control register 2

The following bits of this register perform various functions detailed

with the register bit name.

Register CCR2 Index C2h

Bit map:

----------------------------------------------------------------

7      6      5      4       3        2       1       0

----------------------------------------------------------------

|USE_SUSP| BWRT | Res2 | WT1 |SUSP_HALT|LOCK_NW|USE_WBAK| Res1 |

----------------------------------------------------------------

Bit    Name      Function

7      USE_SUSP  1 = Enable suspend pins

6      BWRT      1 = Enable use of 16 byte write back cycles

5      Res2      Reserved

4      WT1       1 = Write through region 1 (640k..1M)

3      SUSP_HALT 1 = CPU enters suspend mode after HALT instruction

2      LOCK_NW   1 = Prohibit change of state of NW bit of CR0

1      USE_WBAK  1 = Enable cache write back pins

0      Res1      Reserved

USE_WBAK: Enable INVAL and WM_RST input pins, and CACHE# and HITM#

output pins.  Must be set prior to enabling NW bit of CR0.

WT1: Forces all writes to the address region between 640k and 1M

that hit in the on-chip cache to be issued on the external bus.

BWRT: Enable use of 16 byte write back cycles.  Set only if

system logic supports.

-oo-

Register: Configuration control register 3

The following bits of this register perform various functions detailed

with the register bit name.

Register CCR3 Index C3h

Bit map:

-------------------------------------------------------------------

7       6       5       4        3        2       1       0

-------------------------------------------------------------------

|MAP_EN3|MAP_EN2|MAP_EN1|MAP_EN0 |SMM_MODE|LINBRST|NMI_EN|SMI_LOCK|

-------------------------------------------------------------------

Bit    Name     Function

7      MAP_EN3  = 0

6      MAP_EN2  = 0

5      MAP_EN1  = 0

4      MAP_EN0  1 = All configuration registers are available

3      SMM_MODE 1 = SL device compatible mode

2      LINBRST  1 = Linear address sequence is used

1      NMI_EN   1 = NMI is enabled during SMM

0      SMI_LOCK 1 = SMM register lock

SMI_LOCK: Once set can only be cleared by asserting the RESET pin.

LINBRST: Enable linear address sequence.  0 = 1 + 4 sequence, Intel

compatible.

-oo-

Register: Configuration control register 4

The following bits of this register perform various functions detailed

with the register bit name.

Register CCR4 Index E8h

Bit map:

----------------------------------------------------------------

7       6       5       4        3       2       1       0

----------------------------------------------------------------

|CPUIDEN|  Res | FP_FAST| DTE_EN| MEM_BYP| IORT2 | IORT1 | IORT0 |

----------------------------------------------------------------

Bit    Name     Function

7      *CPUIDEN 1 = Enable CPUID instruction and EFlags bit 21

6      Res      Reserved

5      *FP_FAST 1 = Fast FPU exception reporting

4      DTE_EN   1 = Enable directory table entry cache

3      MEM_BYP  1 = Memory read bypassing is enabled

2      IORT2

1      IORT1

0      IORT0    I/O recovery time 0..128 bus clocks 2^x

* = Reserved function

CPUIDEN: Step level 1 up only.

IORT: 0 = no clock delay ... 7h = 128 clock delay.

-oo-

Usage and optimisation:

In setting the CPU please note the the results are very dependent on

the correct setup of the CMOS parameters.  Particular attention should

be placed on the L1, L2 cache settings as well as memory read and

write wait states.  It is suggested that these setting be optimised

first before attempting to "tune" the CPU performance control register.

There maybe some interaction of the CMOS settings and CPU settings

requiring iteration to find the most suitable.

The Cyrix 5x86 as other Cyrix processors requires that the motherboard

and BIOS support the on chip cache (L1) as well as other signal pins.

For this reason the motherboard jumpers and CMOS must be set correctly.

Before starting it is a good idea to run 5x86 /e to see what register

bits have been set by the BIOS.

The cache is enabled/disabled by setting CPU CR0 register which can

only be accessed (written) when the CPU is operating at IOPL 0 or ring

0.  This feature is not to be used in protected mode (32 bit programs)

such as Windows, Unix etc.  No checking is done at present although

possible.  The user is expected to read and understand this statement

and not foolishly attempt to modify the cache setting when operating

the CPU at any other level than 0.  The use of this feature is only

included for those motherboards without Cyrix cache support as a test

at present.  Note running EMM386.EXE and similar memory managers places

DOS at IOPL 3 and you will not be able to change the setting of CR0.  A

clean DOS boot is absolutely essential to use ICD, CWT and CWB.

CBW will enable the write back pins of the CPU and if these are not

supported by the hardware, writeback mode can not be expected to work.

See device driver, cx5x86.exe

Some problems have been noted with LSSER set to 0 the default when

other functions (bits) have been enabled.

Suggested BIOS default values are:

CCR2  BWRT    = 0

CCR4  MEM_BYP = 1

DTE_EN  = 1

FP_FAST = 0

IORT    = 0

PCR0  RSTACK  = 0

BTB     = 0

LOOP    = 0

LSSER   = 1 for PCI memory mapped devices

LSSER   = 0 for systems without PCI

The parameters for use can be found by entering h or ? on the command

line.  When the optimum parameters have been found path\5x86 {parameters}

may be placed in your autoexec.bat file with those parameters so that

the PC is set on startup.

-oo-

What are we dealing with?

Memory management:

Load-store reordering; that prioritises memory reads required by the

integer unit over writes to external memory.

Memory-read bypassing; that eliminates unnecessary memory reads by using

valid data still in the execution unit.

Branch prediction:

The branch prediction target buffer (BTB) is used to store branch target

addresses and branch prediction information.

The target address of a RET instruction is dynamic rather than static

and return addresses are cached in a return stack rather than the BTB.

The return address is pushed on the return stack during a CALL

instruction and popped during the corresponding RET instruction.

-oo-

The device driver : CX5X86.EXE

Use in config.sys file as the FIRST device loaded.  It is of no use at

all if loaded after EMM386 and similar programs.

Usage: DEVICE=[Path\]CX5X86.EXE <[/CD|/WT|/WB]>

Where /CD = Cache disable

/WT = Cache enable in write through mode

/WB = Cache enable in write back mode

The syntax is exact; "/" must be used to proceed the option

and only the options as given are valid.  Upper or lower case

characters may be used.  Only one option (the first) on the

config.sys "device = CX5x86" line is valid.

EXAMPLE:Device=c:\util\cx5x86.exe /wt

(Write through mode selected)

Write through mode sets the CR0 register and LOCK_NW bit of CCR2.

Write back mode sets the CR0 register and USE_WBAK and LOCK_NW bits

of CCR2.  This will cause problems if the hardware does not support

the write back configuration.  Your motherboard supplier or

manufacture should be able to answer this question.  If not are you

buying from the right company?

Cache disable sets CR0 bit CD.

This device driver is not installed and thus takes up no memory

after use.

-oo-

References:

Cyrix 5x86(TM) Microprocessor Data Book

IBM 5x86 BIOS Writers Guide

CTCHIPZ configuration files

Functionality, type and rights:

5x86.EXE is not crippled or contain nagware and relies completely on

the honesty of the user.  Use of the program is not free and the user

is both legally and morally bound to registration and payment of the

registration fee.

5x86 is shareware, all rights are reserved by Peter N Moss.

Copyright:

5x86.EXE and 5x86.DOC are copyright 1995 Peter N Moss.

Disclaimer:

No claim is made for the fitness of purpose of this software and no

claim for damages of any nature will be accepted.  The program is

for use at entirely your own risk.

Distribution:

You may distribute the files only on condition that they are not

altered and all files are included.  The latest version can be found

at ftp://compnt.cs.unp.ac.za/uploads/cyrix/5x86(Ver).zip.

Updates to registered users will only be sent via Internet e-mail

without charge.  Disk updates will be charged at the cost of

materials and postage.

Reports and support:

Whilst reports of results or bugs are welcomed, it is not possible to

support questions on how to setup or optimise any motherboards.  You

are referred to the dealer that sold the board and or CPU.  Registered

users can expect free update notification and as much help as can be

given.  Unregistered users can expect the same consideration as they

have given the author.

Thanks:  Mr C Chan, Mr K J Wren and Mr J Stiel for testing and

feedback.  Cyrix Corp for data.  IBM for data.

Trademarks:  Cyrix is a registered trademark of Cyrix Corp.

Cx5x86 is a registered trademark of Cyrix Corp.

IBM is a registered trademark of International Business

Machines.

Payment:

Payment of US$15.00 or the equivalent is such a small amount that

anyone that uses the program continuously should contribute to the

development of it.  Approximately 500 man hours have gone into the

development of the programs.  It will also insure that the program

continues to be developed.

You are offered 30 days free trial for evaluation only.  After that

period has expired and registration or payment has not taken place

then the program must not be used.

Your honesty will ensure the continued development of shareware

programs, in particular 5x86.  No shareware author wishes to donate

his time and effort to people unwilling to appreciate that effort.

Would you offer a similar amount of time or effort to the shareware

author?  If not, please pay the small amount requested.

For money order payment from outside of South Africa please add $4.00.

Payment in cash, cheques or money order may be made to:

P N Moss

P O Box 212144

Oribi

Pietermaritzburg

Rep of South Africa, 3205

When making payment please include your e-mail address if you have one

and your return mail address.

Registration form:  5X86.REG################################################################################