DISKDUPE USER MANUAL

DiskDupe Trial

________________________________________________________________________

Owner's Manual

Version 9.0

Revision 9

Micro System Designs, Inc.

10062 Miller Ave, Suite 104

Cupertino, CA  95014

Phone 408-446-2066

Fax 408-446-2095

- page 1 -

DISKDUPE USER MANUAL

DiskDupe Trial Ver 9.0 Rev 9 (December 1993)

(DD909T.DOC)

(C) Copyright 1988 - 1993 Micro System Designs, Inc.

All rights reserved.

DISKDUPE,  DISKDUPE  PRO,  DISKDUPE AUTO  PRO,  DISKMACHINE,  AUTOLOAD,

RELAY,  RAMBODISK,  CASCADE, TRACK RANGE and MICRO SYSTEM  DESIGNS  are

trademarks of Micro System Designs, Inc. Other brands and product names

are trademarks or registered trademarks of their respective holders.

No  part of this document may be reproduced, transmitted,  transcribed,

stored in a retrieval system, or translated into any language  (natural

or computer) in any form by any means without the written permission of

Micro  System Designs, Inc., except as described in the  following  li-

cense.

- page 2 -

DISKDUPE USER MANUAL

LICENSE AGREEMENT

This  software  is protected by both United States  copyright  law  and

international treaty provisions.  You are granted a limited license  to

use this program on a trial basis for up to 30 days.  To continue using

the program after the trial period, you are required to pay the license

fee.

You are granted a limited license to copy DiskDupe for the trial use of

others  providing  you include the  files  DISKDUPE.EXE,  DISKDUPE.OVR,

DISKDUPE.DAT, and DISKDUPE.TXT in complete and unmodified form and make

it clear to the recipient that this is not free software but payment of

the license fee is required for continued use after the trial period.

! DO NOT DISTRIBUTE REGISTERED COPY !

LIMITED WARRANTY

Micro  System Designs does not warrant that the functions contained  in

this program will meet your requirements or that the program  operation

will be uninterrupted or error free.

Micro  System Designs specifically disclaims all other warranties,  ex-

pressed or implied, including but not limited to implied warranties  of

fitness for any particular purpose and of merchantability.

IN NO EVENT WILL MICRO SYSTEM DESIGNS BE LIABLE TO YOU FOR ANY DAMAGES,

INCLUDING BUT NOT LIMITED TO ANY LOST PROFITS, LOST SAVINGS, COMMERCIAL

DAMAGE OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES ARISING OUT OF  THE

USE  OR  INABILITY TO USE THIS PROGRAM, OR FOR ANY CLAIM BY  ANY  OTHER

PARTY.

GOVERNING LAW

These  statements shall be construed, interpreted, and governed by  the

laws  of the state of California. You agree that this is  the  complete

and  exclusive statement of this agreement which supersedes  any  prior

agreement  or  other communication between us on  this  subject.   Use,

duplication,  or  disclosure  by the U.S. Government  of  the  computer

software  and  documentation in this package shall be  subject  to  the

restricted  rights  under DFARS 52.227-7013  applicable  to  commercial

computer software.

- page 3 -

DISKDUPE USER MANUAL

INTRODUCTION

* PURPOSE *

DISKDUPE is a disk copying program which support 3.5" and 5.25"  floppy

disks in both normal density and high density.

DiskDupe is the probably best program to automate your disk duplicating

needs.

* SPEED *

DISKDUPE DUPLICATES FLOPPY DISKS FAST!

DiskDupe can duplicate disks amazingly fast and without any fuss. Other

programs simply don't measure up to DiskDupe in terms of speed.

DiskDupe  duplicates, formats and compares disks up to 15 times  faster

than DOS! Copies high-density disks in a single drive without swapping!

To  perform  at maximum speed, DiskDupe bypasses DOS and the  BIOS  and

talks directly to your floppy controller.

* SIMPLE, AUTOMATIC, ERROR DETECTION *

DiskDupe  senses  when you put a disk in the drive and  goes  right  to

work.  So you can copy disks without touching the keyboard.  If a  disk

needs  to be formatted, it will be formatted before data is written  to

it.   Data  can be verified after every write operation  ensuring  data

integrity.

DiskDupe provide pull down menu and support mouse!

* BASIC REQUIREMENTS *

- IBM PC, PS/2, PowerPC or Compatible

- DOS 6.0 or later

- 640k RAM

- Hard Disk (option for data swapping)

(or two floppy drives and 300k EMS or XMS)

* WHAT'S NEW IN 9.09 *

- Newly re-written disk read/write/compare routine, 15 times faster!

- 4 color scheme available, user selectable

- No opening screen delay, faster start-up

- Improvement in disk image handling

- 64-bit CRC checksum in disk data read/write/compare, more reliable

- Support slower floppy drive such as IBM PS/2 lines

- Support faster floppy drive such as PowerPC lines

- MAC, MIPS, Alpha & PowerPC disk support!

- Copied disk are special coded to make it read faster.

- Simulataneous disk read/write (require disk controller from MSD)

- page 4 -

DISKDUPE USER MANUAL

INDEX

1. INTRODUCTION

2. INSTALLATION

NETWORK INSTALLATION

FLOPPY DRIVE INSTALLATION

SOFTWARE COMPATIBILITY

3. NAVIGATING THE MENUS

BASIC OPERATION

4. COPYING DISKS

DISK TO DISK

IMAGES

RELAY

CASCADE

RELAY AND CASCADE

5. FORMATTING DISKS

QUICK ERASE

FULL REFORMAT

FORMAT COMPATIBILITY

6. CLEANING

HOW DRIVES GET DIRTY

HOW OFTEN TO CLEAN

CLEANING THE DRIVE

7. DISK CONVERSION

8. OPTIONS

CASCADE PAUSE

DISK MENU

FORMAT OPTION

VERIFY OPTION

COMPARE OPTION

TEMP FILE

AUTOMATIC DISK SENSING

BAD DISK ALARM

9. MAXIMUM SPEED

CLEAN MASTER

FORMAT AND VERIFY OPTIMIZATION

DUPLICATING TIMES

10. BATCH FILES

COMMAND LINE SWITCHES

11. TROUBLE SHOOTING

12. PROBLEMS

SOFTWARE COMPATABILITY

FREQENTLY ASKED QUESTIONS

TECHNICAL SUPPORT

13. DISKDUPE REGISTRATION FORM

- page 5 -

DISKDUPE USER MANUAL

INSTALLATION

HARD DISK INSTALLATION

To install DiskDupe on your hard disk:

1. Type: MD C:\DISKDUPE <Enter> to make a subdirectory for DiskDupe  on

your hard disk.

2. Type: CD C:\DISKDUPE <Enter> to change to that directory.

3. Insert  the  DiskDupe program disk in drive A:, or  the  appropriate

floppy  drive,  and type: COPY A:\*.* <Enter> to copy the  files  to

your hard disk.

4. Type DISKDUPE <Enter> to run DiskDupe.

DiskDupe  searches  the  current  directory  and  your  path  for   its

configuration file DISKDUPE.DAT. You can add the \DISKDUPE subdirectory

to the path statement in your AUTOEXEC.BAT file so that you do not need

to change to the \DISKDUPE subdirectory every time you run DiskDupe.

NETWORK INSTALLATION

Running DiskDupe on a network allows you to take full advantage of  the

multiuser environment and allows you to share image files. As  long  as

only one computer is using DiskDupe at a time, you can install DiskDupe

on the network under the terms of this license agreement. If more  than

one  person  will be using DiskDupe at the same time,  contact  us  for

information on a site license.

If you are the only one using DiskDupe on your network, you can install

it  as described above in the hard disk installation. However,  if  you

would  like  multiple workstations to be able to access  DiskDupe,  the

network  administrator  should install it according  to  the  following

instructions.

1. Copy all the files from the DiskDupe disk into a shared subdirectory

on the network.

2. Copy  the file DISKDUPE.DAT from the shared subdirectory to a  local

subdirectory that DiskDupe will be started from.

3. DiskDupe  searches  the  current  directory and  the  path  for  its

configuration  file DISKDUPE.DAT, which is updated  with  DiskDupe's

current  settings  each time you exit. If DiskDupe cannot  locate  a

DISKDUPE.DAT file, it creates one in the directory from which it was

started.  There  are several ways for a user's configuration  to  be

saved:

- Add the local subdirectory to the user's PATH statement.

- Keep "Use CMOS for A&B" set as "Yes" in the shared subdirectory.

If started with the shared DISKDUPE.DAT, DiskDupe will start  up

- page 6 -

DISKDUPE USER MANUAL

with the A: and B: drives tailored to the specific workstation's

configuration  at the time it was turned on or  rebooted  (drive

settings can still be changed from within DiskDupe at any time).

This  alleviates  the  need  for some  users  to  have  a  local

DISKDUPE.DAT file.

- Start  DiskDupe  from the local subdirectory or create  a  batch

file to do so.

FLOPPY DRIVE INSTALLATION

If  the  computer  doesn't have a hard disk and is not  attached  to  a

network, you can run DiskDupe on a dual floppy disk system if you  have

at  least  300k  of  EMS  or  XMS. It  is  best if you  can  leave  the

DiskDupe disk in one drive and use the other drive for copying.

If  you need to use both drives for copying, start DiskDupe and set  it

up  exactly the way you want it.  Next exit DiskDupe to save the  setup

information.  Then start DiskDupe again and choose an option under  the

Go menu. You can now take out the DiskDupe disk and use both drives for

copying.

NAVIGATING THE MENUS

DiskDupe  supports  both mouse and keyboard use. When you  first  start

DiskDupe,  you'll  see a menu bar across the top of  the  screen,  from

which you can select pull-down menus, either by clicking on each option

with  the  mouse or moving the highlight bar with the cursor  keys  and

pressing  <Enter>. Once the menus are displayed, click on a  menu  item

with  the mouse to select it, or use the Up/Down arrow keys  to  scroll

vertically through each menu and the Left/Right arrow keys to move from

menu  to  menu. Once you have made the desired changes in  a  box,  the

<Esc> key will get you out of any menu.

Although    the   mouse   and   keyboard   functions   are    generally

interchangeable,  you  may have to use <Enter> or <Esc>  to  select  or

cancel some settings that are specified in menu boxes.

Note:  When  using  the  keyboard, you must press  <Enter>  to  make  a

selection. If you scroll down a menu, and then move to a different menu

without  making  a  selection, the highlight bar  will  remain  on  the

setting  you scrolled down to, but the highlighted setting will not  be

selected.

BASIC OPERATION

DiskDupe  has  three basic operations -  duplicating,  formatting,  and

comparing - each of which is selected from the Go menu.

Duplicate  makes a copy of a disk, Format prepares a new disk for  use,

and  Compare  checks that two or more disks are identical.  The  Copies

setting controls the number of copies that are made. Enter 0 for copies

to  copy continuously. (NOTE: The Trial edition only allows  5  copies,

- page 7 -

DISKDUPE USER MANUAL

and Lite Edition allows 10. All other versions have no limit)

Whether you are Duplicating, Formatting, or Comparing, DiskDupe  always

works  from the Source drive to the Destination drive. It  copies  from

the source disk, sometimes called the master disk, to the  destination,

or  target disk. Sometimes, for instance when you format a  disk,  only

the Destination setting is used.

Although  the simplest way to use DiskDupe is to copy from one disk  to

another  disk,  other  modes are useful for certain  tasks.  Two  other

common  ways of working with disks in DiskDupe are Relay  and  Cascade.

There are also variations, such as creating an image of a disk on  your

hard  disk and making copies from that file. For each variation,  there

are  settings that control how each of the basic operations  work.  The

basic technique goes like this:

1. Try  working from right to left when changing menu  settings.  Start

with  the Options menu, and then check the menu settings  for  Disk,

Destination,  and Source. For the Disk setting, you can usually  use

Auto. DiskDupe then automatically sets the type of disk.

2. From  the Go menu, decide how many Copies you want or the number  of

times  you  want  something done. In some  cases,  this  setting  is

ignored.  (NOTE:  The Trial edition only allows 5 copies,  and  Lite

Edition allows 10. All other versions have no limit)

3. Select the operation, usually Duplicate, Format, or Compare.

COPYING DISKS

Whether  you're duplicating, formatting, or comparing, many  variations

are  possible  in  DiskDupe. Copying from one disk to  another  is  the

simplest  way to make new disks. Variations of this include  Relay  and

Cascade introduced in detail in this chapter.

Regardless  of  the options chosen, DiskDupe makes an exact copy  of  a

disk,  including  hidden and system files, subdirectories,  and  volume

labels.  However,  DiskDupe cannot make duplicates  of  copy  protected

disks.

DISK TO DISK

The  most straightforward way to duplicate is to copy from one disk  to

another. This is useful for making a specific number of copies of disks

of  different types and sizes. For example, if you want 5 copies  of  a

disk, you would select Duplicate with Copies set to 5.

You  do not explicitly need to set up DiskDupe to copy "disk to  disk".

Itis the standard way it works. Below are step-by-step instructions.

1. Move  to  the  Type  menu. Use the <Up> and  <Down>  arrow  keys  to

highlight  the  type of disk you want to copy, or select  Auto,  and

press <Enter>.

- page 8 -

DISKDUPE USER MANUAL

2. From  the  Destination menu, select the drive to which you  want  to

copy.

3. Under Source, indicate the drive from which you want to copy. Source

and Destination can be the same drive, but do not need to be.

4. Move  to the Go menu and set the number of copies that you  want  to

make.  If  you want DiskDupe to copy continuously  until  you  press

<Esc>,  enter 0 for the number of copies. (NOTE: The  Trial  edition

only allows 5 copies, and Lite Edition allows 10. All other versions

have no limit)

5. Highlight Duplicate and press <Enter>.

DiskDupe will prompt you to put the master disk in the Source drive and

the  drive  light will turn on. As soon as you close  the  drive  door,

DiskDupe  will start reading the disk into your computer's RAM  memory,

using  your  hard  disk as virtual memory  if  necessary.  This  allows

DiskDupe  to make as many copies as you want without having  to  reread

the master disk.

Notes:

- 360K  disks :

Although you can duplicate 360k disks in a 5.25", 1.2Mb drive, it  is

not recommended, particularly if the disks might need to be read in a

360k drive. A 1.2Mb drive writes a narrower track than a 360k  drive,

so it does not make a very good 360k duplicate.

- Copy protected disks:

DiskDupe  will not successfully copy disks that are  copy  protected.

However, it will allow you to make copy protected disks of your own.

- Non-MS-DOS disks:

Although DiskDupe is DOS-based, it will allow you to duplicate disks

of  other formats, including Mac high-density, MIPS, Alpha, PowerPC,

Xenix, Unix, and Midi disks. DiskDupe can copy Unix disks that have

an interleave of 3, but the copies it makes will have an interleave

of 1.

IMAGES

If you frequently make copies of one disk, making an image of the  disk

on your hard drive saves time and avoids unnecessary wear on the  disk.

The only drawback is the hard disk space that images take up.

To create an image, set Source to the drive with the master disk in it.

Set  Destination to File and then type in a file name. DiskDupe  images

usually use the extension .DDI. Then select Go, Duplicate to create the

image.

To  use the image, set Source to file, and press <Enter> when  you  see

the *.DDI. DiskDupe will then show you all the images you have created.

- page 9 -

DISKDUPE USER MANUAL

Select the one you want and press <Enter>. Then set Destination to  the

drive you want to copy to and select Go, Duplicate to make the copies.

RELAY

Relay  is an extremely efficient method of backing up program disks  or

making  copies of a shareware library. To use it, first make  sure  all

your  master  disks have write-protect tabs on them. Relay  senses  the

write-protect tabs and automatically reads and copies the master  disks

to  all  the  non write-protected disks after each  master  disk.  This

allows you to make copies of different master disks of one type without

using the keyboard.

Then  set Source to Relay, Destination to A: and select Duplicate  from

the  Go  menu. Put in the first master disk and let DiskDupe  read  it.

Afterwards, put in as many disks as you want to copy. When you want  to

start copying from the next write-protected master, just insert it into

the  drive. DiskDupe will sense the write-protect tab and read the  new

master disk without a keystroke.

1. To  use  Relay, make sure all your master disks  have  write-protect

tabs on them.

2. For  Destination,  select the drive to which you want to  copy.  For

Source select Relay.

3. Under Go, select Duplicate. The number of copies doesn't matter when

using Relay.

4. Put  in the first write-protected master disk and let DiskDupe  read

it.

5. Insert the disks you want the master copied to.

6. When  you are ready to start using the next  write-protected  master

disk  just put it in. DiskDupe will recognize the write-protect  tab

and start reading this in as the next master disk.

7. When you are finished, press <Esc>.

Caution:  If your master disks do not have write-protect tabs on  them,

they will be overwritten.

CASCADE

If you have more than one floppy drive of the same type and are  making

multiple copies, you can use Cascade to get more efficiency out of your

drives.  Using Cascade is similar to copying from A: to B: - reading  a

disk  from one drive and making a copy in another - except  that,  once

the  first  disk is read, DiskDupe cycles through and makes  copies  in

each drive.

- page 10 -

DISKDUPE USER MANUAL

1. Choose Cascade from the Destination menu.

2. Choose the Source drive or Image.

3. Put a write-protected master disk into the first drive.

4. Select Duplicate from the Go menu.

DiskDupe  will read the first disk and then wait for you to remove  the

disk  and  insert new ones in all the drives. DiskDupe will  sense  the

disk  change,  make a copy in the first drive, and then switch  to  the

next drive to make a copy. It will make copies until it reaches the end

of  the cycle, where the Cascade Pause feature waits for you to  change

the disks in all the drives. DiskDupe will sense when you insert a  new

disk in the first drive and resume copying immediately.

DiskDupe  will continue copying until the desired number of copies  has

been  made or until you hit <Esc> to break the cycle. If a bad disk  is

encountered,  you will have to remove it and insert a new disk in  that

same drive before DiskDupe resumes duplicating.

Note:  The  IBM  PC and compatibles cannot copy to  two  drives  simul-

taneously.

RELAY AND CASCADE

You  can  also combine Cascade with Relay. In such a setup,  the  first

write-protected  master disk is read from the first drive in the  cycle

and  will be copied to every non write-protected disk inserted in  that

drive.  In the second drive another master disk is read and  copied  to

each non write-protected disk inserted in that drive.

FORMATTING DISKS

With  DiskDupe, you can either format a disk as it is being  duplicated

or separately, by choosing Format from the Go menu. When you copy disks

with Duplicate, DiskDupe will detect whether a disk has been  formatted

and,  if  necessary,  format it while copying.  Format  is  a  separate

feature  used whenever you want to just format some disks or  preformat

disks ahead of time for later use with DiskDupe. To format disks:

1. Go to the Options menu to select the Format and Verify settings. The

options are discussed below.

2. If you are formatting a 720k disk in a 1.4Mb drive, set Disk to 720k

otherwise leave it set to Auto.

3. Set  Destination to the drive you will use to format the disk.  This

can  be  Cascade  to  use  both  drive.  Source  is  not  used  when

formatting.

4. Select Format from the Go menu.

- page 11 -

DISKDUPE USER MANUAL

QUICK ERASE

To  do  a  quick erase, set Format under the Options menu  to  Only  If

Necessary and set Verify to Only After Format.

Reformatting a disk that has been previously formatted takes only about

2  seconds per disk. This effectively wipes out the data on a  disk  by

writing  over  the  disk's FAT (File Allocation  Table)  and  directory

table.  DiskDupe will write the first couple of tracks, where the  file

and  directory  information is located, realize that the  disk  doesn't

need to be formatted, and prompt you for another disk. If you run a DIR

or CHKDSK on the disk, you will see that the disk is indeed empty.

If you also want to recheck the disk to make sure there are no  damaged

areas,  set  the Verify option to Always. This is quicker than  a  full

reformat, but will check the entire disk for defects.

Note:  Although  the  disk appears blank, some of  the  data  is  still

physically  on the disk. The only time this might pose a problem is  if

the  disk  contains sensitive information that you want to be  sure  to

wipe off completely. If so, use the full reformat option.

FULL REFORMAT

To do a full reformat, set Format under the Options menu to Always.

Use  full reformat if you plan to use an old disk as a master  disk  or

want to completely erase all the data on it.

FORMAT COMPATIBILITY

When DiskDupe formats disks using Format under Go, it places a DOS  3.3

boot track on the disk. This is compatible with all versions of DOS. If

you  want  to format a disk with the boot track of another  version  of

DOS,  or make a bootable disk or make formatted Unix, Midi or Mac  high

density disks, follow these instructions.

1. Format a disk using the normal format command for that system.  With

DOS this is FORMAT or FORMAT /S.

2. Use  DiskDupe to make a copy of this disk. If you want you can  also

create an image file of this disk and use that as the Source.

If  you  are  using a master disk formatted with  DOS  4.0  or  higher,

DiskDupe  will  copy  the  serial number of  the  master  disk  without

changing it.

CLEANING OF DRIVE HEAD

To ensure good copies and maximum data integrity , it is critical  that

the  drive heads be kept clean. Dirty heads can cause good disks to  be

rejected and bad disks to be passed.

- page 12 -

DISKDUPE USER MANUAL

HOW DRIVES GET DIRTY

Drive  heads get dirty in two ways: (1) Particles on the  disk  surface

like dust and cigarette smoke can stick to the head and/or the magnetic

coating  on  the  disk rubs off on the head.  These  coatings  vary  in

quality and they all absorb moisture.  (2) Poor quality disks that have

been in humid environments can clog the drive heads after just 50 or 60

disks.

When the head gets dirty, it no longer makes good contact with the disk

and  the dirt particles diffuse the reading and writing  process.  This

produces  erratic  results.  Some  disks work  fine  while  others  are

inexplicably rejected.

HOW OFTEN TO CLEAN

We  recommend  that you clean the drive heads every 500  disks,  or  at

least once a month.  Sometimes in humid or dirty environments, you will

need  to clean the heads every 100 disks.  If you start experiencing  a

lot of rejected disks or have other problems, it is always a good  idea

to clean the heads.

When  drives are extremely dirty, you can actually see the dirt on  the

head, which are reddish-brown spots on the white ceramic piece attached

to the moveable head-frame.

CLEANING THE DRIVE

To  clean  the  drive, use a good wet-dry  type  cleaning  disk.  Under

DiskDupe's  Destination menu, select the drive you want to clean.  Then

choose  Clean from the Go menu. Insert the cleaning disk in  the  drive

and press <Enter>. The drive will be cleaned for 15 seconds.

While cleaning, DiskDupe moves the head back and forth across the disk.

This  provides the best possible cleaning and makes sure  the  cleaning

disk  is evenly used. Be sure to discard the cleaning disk if  you  see

any discoloration on it or it has been used past the recommended number

of cleanings.

If you clean your drives regularly, a 15 second cleaning is sufficient.

However, if your drives are especially dirty, several cleanings may  be

required. In extreme cases, it is necessary to clean the drive head  by

hand with a Q-tip and isopropyl alcohol.

Cleaning  the drive does not wear it out. A dirty drive  will  actually

wear out faster because of the abrasive action of the dirt.

DISK CONVERSION

DiskDupe  allows  you  to convert certain types of disks  to  disks  of

another  physical size, such as a 5.25", 1.2Mb disk to a  3.5",  1.4Mb.

- page 13 -

DISKDUPE USER MANUAL

You  can,  for example, copy a program's 5.25" program  disks  to  3.5"

disks for installation on a laptop which only has a 3.5" drive.

Converting  disks  is  just  a special  application  of  the  Duplicate

command. The table below indicates which disks, drives, and settings to

use  for  a  conversion.  The conversions  listed  are  the  only  ones

presently supported.

Conversion        Disk type      Drive Type

-------------     ----------     -------------------

360k to 3.5"      720k disk      720k or 1.4Mb drive

720k to 5.25"     360k disk      1.2Mb drive

1.2Mb to 3.5"     1.4Mb disk     1.4Mb drive

For the 3.5", 720k to 5.25" conversion, copy to a low-density 360k disk

in  the 1.2Mb drive. If you get an error on several disks, try using  a

1.2Mb  disk  instead. This disk will be formatted to 720k  capacity  by

using  the area between the tracks. If a computer has problems  reading

the files on the disk, even though it recognizes the disk and can do  a

DIR of it, use the FIX720.COM program. This is a small  memory-resident

program that allows such computers to read this 5.25" disk.

(NOTE: FIX720 is included with the registered versions of DiskDupe.)

DiskDupe  transfers  the information track-by-track,  including  volume

labels,  subdirectories, hidden and system files. During a  conversion,

the  destination  disk will be formatted to the same  capacity  as  the

source  disk. So if you are copying from a 5.25", 360k disk to a  3.5",

720k disk, the 3.5" disk will have a capacity of 360k.

Track-by-track  conversion won't copy from a higher density disk  to  a

lower  density  disk, even if the higher density disk has  just  a  few

files  on it. In this case, it's better to use the DOS Copy command  to

transfer the files.

Note: If you are copying disks to send to others, we recommend making a

master  for each disk size, rather than using track  conversions.  This

allows you to use the full capacity of the disk and will alleviate  any

possible problems that other computers might have reading these disks.

OPTIONS

CASCADE PAUSE

If you intend to change each disk as it is copied, you can set  Cascade

Pause  to Off under the Options menu. DiskDupe will cycle  continuously

and overwrite the disks in the drives, including the master disk if  it

is not write-protected. If you prefer to change the disks all at  once,

leave Cascade Pause set to On, and DiskDupe will wait for you to do so.

Example

Using  Cascade  with drives A: and B:: While the master disk  is  being

- page 14 -

DISKDUPE USER MANUAL

read  from  drive  A:, you put a blank disk in drive  B:.  As  soon  as

DiskDupe  is finished reading from drive A:, it will start  writing  to

the disk in drive B:. If Cascade Pause is set to On, DiskDupe will wait

for you to hit a key at the end of each cycle, indicating that you have

replaced all the disks. With Cascade Pause set to Off, you must remove,

and  replace, the disks as they are copied or DiskDupe  will  overwrite

them.

DISK MENU

Normally  you can leave this set at AUTO to let DiskDupe  automatically

sense the type of disk you are using. When you are formatting and doing

diagnostics  on a 1.4Mb drive, DiskDupe will assume you want to  use  a

1.4Mb disk. If you want to use a 720k disk, you will need to select  it

manually.  The same is true for a 360k disk in a 1.2Mb drive.  Also  if

you are copying non-DOS disks like Unix, Midi or Mac high density,  you

will also need to select the copy type manually.

NOTE: Although you can duplicate 360k disks in a 5.25", 1.2Mb drive, it

is not recommended. The 5.25", 1.2Mb drives write a smaller track  than

usual so they do not make very good 360k duplicates.

FORMAT OPTION

The  Format  and  Verify options are used to help  speed  up  the  disk

duplicating time. If you always use new disks when making copies, go to

FORMAT  under the OPTIONS menu, press <ENTER> and then  select  ALWAYS.

This  will save a few seconds that DiskDupe normally takes to  identify

that a disk is blank and shift into Format mode. When Format is set  to

ALWAYS, a 360k disk will take less than 58 seconds to duplicate.

If,  however, you sometimes use disks that are pre-formatted,  you  can

save  a lot of time by setting FORMAT to ONLY IF NECESSARY . This  will

skip the format step on disks that have been pre-formatted. You can use

this  option  even if you use a combination of  pre-formatted  and  new

disks  because  DiskDupe  will  still sense when  a  disk  is  new  and

automatically shift into format mode for that disk.

VERIFY OPTION

If  you  are confident that the disks you have pre-formatted  have  not

been  subsequently  damaged,  you can also set  VERIFY  to  ONLY  AFTER

FORMAT.  On pre-formatted disks this can cut the duplicating time on  a

360k disk down to less than 16 seconds per disk.

Be aware, however, that if the disks have been damaged after they  were

formatted  (for  instance, someone accidently touched the  floppy  disk

surface),  DiskDupe will probably not detect this damage if  VERIFY  is

set  to  ONLY AFTER FORMAT. Duplicating times for  pre-formatted  disks

when Verify is set to ALWAYS are still fairly quick.

- page 15 -

DISKDUPE USER MANUAL

COMPARE OPTION

To compare a disk byte-by-byte to the master during verify, turn On the

Compare option under Options menu.

Compare provides a more thorough check than a Verify. You can run  both

together if you are running DiskDupe on a 386 or later model  computer.

Using  both Verify and Compare will not slow down the  duplication  any

more  than running just one. Under Options, select On for  Compare  and

set Verify to Always.

On  some  286's  and  slower computers,  compare  during  verify  might

slowdown  the  copying speed. If your computer passed  the  diagnostics

tests  under  the  Go menu, you can turn Compare Off  and  still  catch

99.97% of all disk errors.

TEMP FILE

The  Temp File is the file on your hard disk where DiskDupe stores  any

part  of  the disk image that will not fit in memory. If  you  have  at

least  640k  of RAM memory in your computer and  are  duplicating  360k

disks,  the  entire  disk will fit in memory and will not  have  to  be

buffered. If however, you are duplicating another type of disk or don't

have as much memory, part of the disk will be buffered to this file.

DiskDupe  will  only store the parts of the disk with data on  it.  For

instance,  if you are duplicating a disk that is only half  full,  only

that half of the disk that has data on it will be stored.

Note:  If  files have been erased from a disk, DiskDupe may  be  fooled

into thinking that there is more data on the disk than there really is.

To  help  DiskDupe  work most efficiently, it is best to  use  a  newly

formatted disk as your master disk.

OTHER FEATURES

AUTOMATIC DISK SENSING

DiskDupe  automatically senses when you change disks without having  to

touch  the  keyboard.  This will speed up  your  duplication  and  also

eliminate errors.

To  sense a disk change, DiskDupe needs to keep the drive on.  So  when

DiskDupe  alerts  you, it is okay to change the disk  even  though  the

drive light is still on.

BAD DISK ALARM

DiskDupe sounds an alarm when it encounters a bad disk and asks you  to

press the <ENTER> key. This alerts you that there is a bad disk to make

- page 16 -

DISKDUPE USER MANUAL

sure you don't accidentally put it in the good disk pile.

MAXIMUM SPEED

There  are several ways you can optimize the speed of the  copies  made

with DiskDupe.

CLEAN MASTER

DiskDupe works faster when the master disk is "clean". This means  that

the master disk was formatted before any files were copied to it.

Using  a clean disk, DiskDupe is able to sense which part of  the  disk

has  valid data on it and only duplicate that part of the disk. If  you

have a master disk which has had many different files copied to it  and

then  erased,  DiskDupe  will have to duplicate the  entire  disk  even

though all of the disk might not contain real data.

To make a clean master disk, first format a disk. Then use the DOS COPY

command  to  copy the necessary files onto this newly  formatted  disk.

This disk can now be used as your clean master disk.

FORMAT AND VERIFY OPTIMIZATION

1.   Pre-format  the  disks.  This  can be done  by  copying  from  a  blank

formatted  disk. The disks will be formatted and verified and  any  bad

disks  will  be  rejected. You can format disks at  any  time  that  is

convenient.

2.   Use  these  pre-formatted disks later when you make copies.  Make  sure

that FORMAT is set to ONLY IF NECESSARY and VERIFY is set to ONLY AFTER

FORMAT on the SETUP menu.

3.   If you are nervous about not verifying the disk after you write to  it,

set  the  VERIFY option to ALWAYS. This will increase  the  duplicating

times but will re-verify the entire disk.

DUPLICATING TIMES

The  following  table  shows how long it takes  DiskDupe  to  duplicate

disks. If the source disk is not completely full, the duplicating times

will be faster than this.

Disks copied in one hour using DiskDupe. All these times assume that it

takes two seconds to change the disk.

360K     1.2Mb    720K     1.4MB

-----    ------   -----    -----

Blank Disks             62       38       32       32

Pre-formatted/verify   106       65       55       55

Pre-formatted          200      126      106      106

- page 17 -

DISKDUPE USER MANUAL

Note: Some 1.4Mb drives however, won't be able to achieve the full  106

disk per hour.

Notice how much time you can save if you pre-formatted disks  yourself.

However, if you buy disks that have already been formatted for you,  we

recommend  that you set Verify to Always. This insures that  any  disks

that are bad will be detected.

BATCH FILES

All DiskDupe parameters can be entered from the DOS command line.  This

allows you to create DOS batch files to automate the use of DiskDupe.

COMMAND LINE

You  can  enter all the information on the DOS command  line  that  you

would  normally have to enter by using the DiskDupe menus. Here  is  an

example:

DISKDUPE A: B: /C5 /DUP

This  will  start DiskDupe with A: as the source drive and  B:  as  the

destination drive and will make 5 copies. The /DUP statement  instructs

DiskDupe  to start duplicating and then return to DOS without  stopping

at  the  menus. This is useful if you want to use DiskDupe in  a  batch

file.

The A: is the Source and can be a drive, a filename or the word RELAY.

The  B:  is the Destination and can be a drive or a  filename.  To  use

Cascade drives, use AB: like DISKDUPE D1 AB:

The /C5 sets the number of copies to 5.

The  /C is called a switch. There are many different switches  you  can

use.  You  can  use as many or as few switches as you  want.  The  next

section lists all the switches.

COMMAND LINE SWITCHES

/DUP      Starts  duplicating without pausing at the menus and  returns

to DOS when finished.

/FOR      Starts formatting without pausing at the menus and returns to

DOS when finished.

/CLEAN    Starts  cleaning without pausing at the menus and returns  to

DOS when finished.

/TEST     Starts  the diagnostics without pausing at the menus and  re-

- page 18 -

DISKDUPE USER MANUAL

turns to DOS when finished.

/DAT      Use alternate .DAT file for different duplicating jobs.

Example /DATJOB1.DAT

/C        Allows you to set the number of copies

Example:  /C5  Set 5 copies. (NOTE: The  Trial  edition  only

allows  5 copies, and Lite Edition allows 10. All other  ver-

sions have no limit)

/FA       Sets the option for formatting to ALWAYS.

/FO       Sets the option for formatting to ONLY WHEN NECESSARY.

/VA       Sets the option for verifying to ALWAYS.

/VO       Sets the option for verifying to ONLY AFTER FORMAT.

/T        Sets the Disk type.

Auto Specifies Auto

360 Specifies a 5.25" 360K copy (Low Density)

12 Specifies a 5.25" 1.2M copy (High Density)

720 Specifies a 3.5" 720K copy (Low density)

14 Specifies a 2.5" 1.4M copy (High Density)

Example: /T720 Specifies a 720k copy.

/S        Sets the type of installed drive.

For drive, use one of the following:

A:, B:

For setup information, use one of the following:

360, 12, 720, 14

Example: /SA:12 Specifies the A: drive is a 1.2M High Density

drive

/CMOS     "YES"  uses CMOS bootup configuration for A: and  B:  drives,

"NO" does not. Example: /CMOSYES

/TF       Allows  the  user to set the option for the  temporary  file.

Example:   /TFDISKDUPE.TMP   Sets  the  temporary   file   to

DISKDUPE.TMP

BATCH FILE EXAMPLE

You  can  combine DiskDupe commands into DOS batch  files  to  automate

sequences of tasks.

The following is a batch file that reads in a source disk and creates a

disk image file and makes 10 duplicates.

DISKDUPE B: DISK1 /DUP

IF ERRORLEVEL 1 GOTO END

DISKDUPE DISK1 B: /C10 /DUP

:END

- page 19 -

DISKDUPE USER MANUAL

Notice  that you can test if there was a problem using  the  ERRORLEVEL

command.  An ERRORLEVEL of one or greater indicates that there  was  an

error.

- page 20 -

DISKDUPE USER MANUAL

TROUBLE SHOOTING

SOFTWARE COMPATIBILITY

- Windows 3.1: If you receive an error message saying the DMA buffer is

too small, add the line DMABufferSize = 018 to the Windows SYSTEM.INI

file in the [386Enh] section.

- QEMM  and SmartDrive 3.1: If you receive an error message saying  the

DMA buffer is too small, change your CONFIG.SYS to "QEMM386.SYS  DB=3

DMA=15" to load QEMM. You don't need the DB=3 parameter if you  don't

have a SCSI hard disk.

- OS/2:  For  maximum  speed, open the DOS settings box  for  the  DIS-

KDUPE.EXE  object.  You'll need to set HW_TIMER to Off and  IDLE_SEN-

SITIVITY to 100 for the DOS session.

FREQUENTLY ASKED QUESTIONS:

PROBLEM: After I copy a disk using DiskDupe and then try to do a DIR on

it, I get an error reading the disk.

ANSWER: Take the disk out of the drive. Type <CTRL-C> several times and

put the disk back in.

QUESTION:  On  my PS/2 it copies the first disk fine but then  makes  a

grinding noise and sometimes I get a "Bad Seek Error".

ANSWER:  Some  of  the PS/2 drives can't step at the  proper  rate.  To

compensate  for this, under Setup tell DiskDupe that it is a  720k

drive.  You will still be able to copy both 720k and  1.4Mb  disks

but you won't have the stepping problem.

QUESTION: I am trying to copy 360k disks on my AT type computer and  it

is not working.

ANSWER: Make sure you have your A: drive set up correctly (usually as a

1.2Mb drive) and that the copy type is set to 360k.

TECHNICAL SUPPORT

DiskDupe is backed by a technical support system designed to offer

you   fast,  courteous  service.  All  registered  users   receive

unlimited, free technical support. We also provide limited support

to unregistered users.

In  order for us to provide the best possible help please  do  the

following  before  calling. Write down the serial  number  of  the

DiskDupe program, write down exactly what happened and what  error

message  appeared, write down what type of computer you are  using

and be at your computer with DiskDupe running.

- page 21 -

DISKDUPE USER MANUAL

MSD PRODUCTS LINES

DISKDUPE TRIAL

Duplicates  and formats 5.25" and 3.5" up to 15 times faster  than

Dos. Copies high density disks in one pass with no swapping. Talks

directly to the floppy controller for maximum speed. Uses the hard

disk  as  virtual memory. Automatically formats disks  if  needed.

Senses disk changes without a keystroke. Detects all bad disks for

perfect  copies.  Stores images on hard disk. Converts 5"  to  3".

Automatically  detects  drive and disk type. Has  Relay,  Cascade,

Diagnostics  and Clean. Uses command line or pull-down menus  with

mouse support.

DISKDUPE LITE

Same  features  as  the  Trial edition  but  removes  the  opening

screens,  increases the copy limit to 10 and gives you  access  to

unlimited, free technical support. The manual is on the disk.

DISKDUPE STANDARD

Adds  ability  to  compare disks byte-by-byte,  choice  of  screen

colors, no limit on the number of copies made and EMS/XMS support.

Includes  printed  manual, re-usable product box  and  free  drive

cleaning kit.

DISKDUPE PRO

Adds  support for up to four floppy drives, double  speed  drives,

2.8Mb  floppies, RamboDisk for fast custom disks, sets  of  disks,

Track Range for faster copying, serialization, label printing  and

copy protection interface.

DISKDUPE AUTO PRO

All  features of DiskDupe Pro plus works with any  XPress,  Trace,

Mountain, Ventuno, Rimage, Ashby, or Costas autoloader for  hands-

free   copying.  Sounds  an  alarm  when  attention   is   needed.

Automatically sends bad disks to the reject bin.

DISKMACHINE

The DiskMachine includes an auto-loader that copies both 5.25" and

3.5"  disks,  DiskDupe Auto Pro, cables and drives in  a  complete

package.

- page 22 -

DISKDUPE USER MANUAL

DISKDUPE REGISTRATION FORM    Send to: Micro System Designs, Inc.

10062 Miller Ave, Suite 104

Version 9.09 Trial                     Cupertino, CA 95014

408-446-2066 Phone

Serial # ____________                  408-446-2095 Fax

YES! I want to register my copy of DiskDupe. (For faster service call or

fax us with your Mastercard or Visa.)

----------------------------------------------------

| DiskDupe Lite 9.0                @   $5  |       |

----------------------------------------------------

| DiskDupe Standard 9.0            @  $15  |       |

----------------------------------------------------

| DiskDupe Pro 9.0                 @  $25  |       |

----------------------------------------------------

| DiskDupe Auto Pro 9.0            @  $35  |       |

----------------------------------------------------

| Sales Tax: CA Residents only             |       |

----------------------------------------------------

| Shipping: US - $2, Int'l - $5            |       |

----------------------------------------------------

| TOTAL                                    |$      |

----------------------------------------------------

Send me DiskDupe on: __5" Disks __ 3" Disks

PAYMENT OPTIONS:

Payment by:   __MC __Visa __Check (US dollars)

__COD (US orders only) __PO (Call for approval)

CREDIT CARD INFORMATION:

Card#_______________________________ Expiration date: ___ /___

Name as it appears on the card________________________________

Signature X___________________________________________________

SHIPPING INFORMATION:

Name__________________________________ Title__________________

Company Name__________________________________________________

Address_______________________________________________________

City_________________________________ State_____ Zip__________

Phone#______________________  FAX#____________________________

- page 23 -